AEC

Part A

1)Simulate to measure the light intensity using Tinkercad

int sensorValue = 0;

void setup()

{

pinMode (A0, INPUT);

Serial.begin(9600);

pinMode(9, OUTPUT);

}

void loop()

{

sensorValue = analogRead(A0);

Serial.println(sensorValue);

analogWrite(9, map(sensorValue,0,1023,0,255));

delay(100);

}


link:

https://www.tinkercad.com/things/gYmeoPsz80l-light-intensity

2) Simulate an ultrasonic distance sensor to detect the distance from the object

long cm = 0;

long readUltrasonicDistance(int triggerPin, int echoPin)

{

pinMode(triggerPin, OUTPUT);  // Clear the trigger

digitalWrite(triggerPin, LOW);

delayMicroseconds(2);

// Sets the trigger pin to HIGH state for 10 microseconds

digitalWrite(triggerPin, HIGH);

delayMicroseconds(10);

digitalWrite(triggerPin, LOW);

pinMode(echoPin, INPUT);

// Reads the echo pin, and returns the sound wave travel time in microseconds *0.01723

return (pulseIn(echoPin, HIGH)*0.01723);

}

void setup()

{

Serial.begin(9600);

pinMode(2, OUTPUT);

pinMode(3, OUTPUT);

pinMode(4, OUTPUT);

}

void loop()

{

cm = readUltrasonicDistance(7, 6);

Serial.print(cm);

Serial.println("cm");

if (cm > 250) {

digitalWrite(2, LOW);

digitalWrite(3, LOW);

digitalWrite(4, LOW);

}

if (cm <= 250 && cm > 175) {

digitalWrite(2, HIGH);

digitalWrite(3, LOW);

digitalWrite(4, LOW);

}

if (cm <= 175 && cm > 100) {

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, LOW);

}

if (cm <= 100) {

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

}

delay(100); // Wait for 100 millisecond(s)

}

Link:
https://www.tinkercad.com/things/5Y6hDvN8QgT-ultrasonic-distance-sensor

3) Simulate a DC motor with specified input that runs continuously and vary with the DC motor speed.

int MPin = 9;   //motor pin

int Analog = A0;  // Analog Input

int Ainp = 0; // Read analog input, initialized to zero

void setup()

{

Serial.begin(9600);

pinMode(MPin, OUTPUT);

pinMode(Analog, INPUT);

}

void loop()

{

Ainp = analogRead(A0);

int Speed = map(Ainp, 0, 1024, 0, 255);

delay(100);

Serial.println(Speed);

analogWrite(MPin, Speed);

}

Link:
https://www.tinkercad.com/things/a1erhbKdSBQ-brave-vihelmo-amur



4) Simulate the door sensor for smart homes using IoT sensor

#include <Servo.h>

Servo myservo;  // create servo object to control a servo

// twelve servo objects can be created on most boards

int poson = 1  ;   // on position(!!YOU WILL HAVE TO SET IT BY TRY AND ERROR ACCORDING TO YOUR SETUP!!)

int posoff =  100 ;  //off position(!!YOU WILL HAVE TO SET IT BY TRY AND ERROR ACCORDING TO YOUR SETUP!!)

int pirPin = 7;    //the digital pin connected to the PIR sensor's output

int ledPin = 13;

int led2Pin=12;

void setup() {

pinMode(pirPin, INPUT);

pinMode(ledPin, OUTPUT);

pinMode(led2Pin,OUTPUT);

digitalWrite(pirPin, LOW);

myservo.attach(9);  // attaches the servo on pin 9 to the servo object

// delay(30000); //enough time for the sensor to calibrate

}

void loop() {

if(  digitalRead(pirPin) == HIGH        ) {

digitalWrite(ledPin, HIGH);

digitalWrite(led2Pin,LOW);

myservo.write(45);              // tell servo to go to position in variable 'poson'

delay(5000);

myservo.write(90);              // tell servo to go to position in variable 'posoff'

digitalWrite(ledPin, LOW);

digitalWrite(led2Pin,HIGH);

delay(1000);   //delay so it wont start right again imidately

digitalWrite(led2Pin,LOW);

}

}

Link:

https://www.tinkercad.com/things/6Dz5LAuc7Q0-glorious-waasa-trug

Part B

1) AI in Agriculture - Smart Irrigation System

2) AI in Smart Cities - Smart Street Lighting with Iot

// Pin definitions

const int ldrPin = A5;     // LDR connected to analog pin A0

const int bulbPin = 3;     // Transistor base controlled by digital pin 9

// Threshold for darkness (adjust based on your setup)

int threshold = 500;

void setup() {

pinMode(bulbPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

int ldrValue = analogRead(ldrPin);   // Read LDR value

Serial.println(ldrValue);            // Print for debugging

if (ldrValue < threshold) {

digitalWrite(bulbPin, HIGH);       // Turn bulb ON

} else {

digitalWrite(bulbPin, LOW);        // Turn bulb OFF

}

delay(200); // Small delay for stability

}

Link:

https://www.tinkercad.com/things/hNq5hOrV0Eh-cool-hango-jaban